# CISCO NETWORKING ASSIGNMENT

## Members

CIT/00108/024:	Laban Wanjala

CIT/00085/024:	Julius Koshal

CIT/00087/024:	Kelvin Kipkosgei

CIT/00050/024:	Evans Muendo

CIT/00031/024:	Margaret Mwende

CIT/00055/024:	Denis Kyalo

CIT/00044/024:	Vincent Kipkurui

CIT/00015/024:	Justin Otiso

CIT/00024/024:	Vivian Nechesa

CIT/00006/024:	George Okoth

## 1.5.7 Network Representation

## Step 1: Identify Common Components of a Network

Question: List the intermediary device categories.

- Routers

- Switches

- Hubs

- Wireless Access Points

- Firewalls

Question: Without entering into the internet cloud or intranet cloud, how many icons in the topology represent endpoint devices (only one connection leading to them)?

Ans:	14

Question: Without counting the two clouds, how many icons in the topology represent intermediary devices (multiple connections leading to them)?

Ans:	6

Question: How many end devices are not desktop computers?

Ans:	10

Question: How many different types of media connections are used in this network topology?
Answer:

- Copper Straight-Through

- Copper Cross-Over

- Fiber

- Console

- Wireless

- Serial DCE/DTE

- Coaxial

- Octal

- IoT Custom cable

- USB

## Step 2: Explain the Purpose of the Devices

Question a: Explain the client-server model.
Answer:
The client-server model is a distributed application structure where:

- Clients (like PCs, laptops, smartphones) request services or resources

- Servers (like web servers, email servers, file servers) provide those services or resources

- Clients initiate communication sessions with servers, which wait for requests

- Examples: Web browsing (client browser requests web pages from web server), email (email client requests mail from mail server)

Question b: List at least two functions of intermediary devices.
Answer:

- Regenerate and retransmit data signals - They ensure data travels long distances without degradation

- Determine paths for data - Routers use IP addresses to determine the best path to destination networks

- Manage data flow - Switches direct frames based on MAC addresses

- Notify of errors and communication failures - They can send error messages back to sending devices

- Provide security - Firewalls and other devices filter traffic to prevent unauthorized access

Question c: List at least two criteria for choosing a network media type.
Answer:

- Distance - How far the signal needs to travel (copper for short distances, fiber for long distances)

- Bandwidth requirements - How much data needs to be transmitted (fiber for high bandwidth, copper for moderate)

- Cost - Budget constraints (copper is generally cheaper than fiber)

- Environmental factors - Interference, physical obstacles, security concerns

- Number of devices - How many nodes need to be connected

## Step 3: Compare and Contrast LANs and WANs

Question a: Explain the difference between a LAN and a WAN. Give examples of each.
Answer:

- LAN (Local Area Network) : A network confined to a small geographic area like a single building, office, or home.

  - Examples: Home Wi-Fi network, school computer lab, office network in one building

  - Characteristics: High speed, privately owned, short distance

- WAN (Wide Area Network) : A network that spans a large geographic area, connecting multiple LANs.

  - Examples: The Internet, company network connecting offices in different cities, bank network connecting branches nationwide

  - Characteristics: Lower speed (typically), uses service providers, long distance

Question b: In the Packet Tracer network, how many WANs do you see?
Answer: [Look for connections between different geographic locations or through the cloud]
Typically: 1-2 WANs (often represented by the "Cloud" or connections between cities)

Question c: How many LANs do you see?
Answer: [Count each separate network segment with switches and local devices]
Typically: 3-5 LANs (like Home Office, Branch Office, Main Office, etc.)

Question d: Briefly describe the internet.
Answer:
The internet is a global network of interconnected networks (a "network of networks") that uses the TCP/IP protocol suite to connect billions of devices worldwide. It includes:

- Millions of public, private, academic, business, and government networks

- No single central ownership or control

- Provides services like World Wide Web, email, streaming, social media

- Uses Internet Service Providers (ISPs) to connect end users

- Relies on routers and extensive cabling (fiber optic, copper) worldwide

Question e: What are some common ways a home user connects to the internet?
Answer:

- DSL (Digital Subscriber Line) - Uses existing telephone lines

- Cable broadband - Uses TV cable lines

- Fiber optic (FTTH) - Fiber to the home (like Google Fiber, Verizon Fios)

- Satellite - For rural areas without cable/DSL

- Cellular (4G/5G) - Using mobile phone networks

- Fixed wireless - Radio signal from a tower

Question f: What are some common methods that businesses use to connect to the internet in your area?
Answer:

- Dedicated leased lines - T1, T3, fiber connections

- Metro Ethernet - High-speed Ethernet over a metropolitan area

- MPLS (Multiprotocol Label Switching) - Private WAN connections

- Business broadband - Higher-tier cable/DSL with SLAs

- Fiber optic connections - Direct fiber connections to the ISP

- Multiple redundant connections - For reliability and failover

# 2.3.7 Navigate the IOS

## Layout

## Findings

2.5.5 CONFIGURE INITIAL SWITCH SETTINGS

QUESTIONS

a. There are 24 fast Ethernet interfaces

b. Switch have 2 gigabit Ethernet interfaces

c. Vty lines range from 0-4 and 5-15

d. Show startup-config display current contents of NVRAM

e. Switch respond to startup-config is not present because no configuration has been saved to NVRAM

f. Enable secret password is used so as the password does not appear in plain text

g. Message of the day(MOTD) Is displayed to anyone trying to login into the switch.

h. Every switch has a MOTD to serve as legal notice to warn unauthorized users that access is restricted

i. Show startup-config display the contents of NVRAM

j. Copy running-config startup-config save all the changes In the

| DEVICE | Interface | IP Address | Subnet Mask |  |
| --- | --- | --- | --- | --- |
| S1 | VLAN 1 | 192.168.1.1 | 255.255.255.0 |  |
| S2 | VLAN 1 | 192.168.1.2 | 255.255.255.0 |  |
| PC 0 | NIC | 192.168.1.10 | 255.255.255.0 |  |
| PC 1 | NIC | 192.168.1.11 | 255.255.255.0 |  |

## 2.7.6 Implement Basic Connectivity

QUESTIONS

1.How can you verify that both passwords were configured correctly?(part 1,step3 )

- Test the Console Password: Type exit to log out; you should be prompted for a password immediately to get back to the s1> prompt.

- Test the Privileged Secret: Type enable at the s1> prompt; it should ask for your secret password before changing to the s1# prompt.

- To Check the Configuration File: Run show running-config and look for enable secret and line con 0 to ensure the passwords are listed in the system settings.

2. Which command do you issue to accomplish this step?(part1,step4)

S1(config)# exit (or end)

S1# copy run start

3. Were you successful? Why or why not?(part2,step2).

My ping should have been unsuccessful because the switches have not been configured with an IP address.

4. If this is the case, why would we configure it with an IP address?(part3,step1)

In order for you to connect remotely to a switch, you need to assign it an IP address. The default configuration on the switch is to have the management of the switch controlled through VLAN 1.

4. Why do you need to enter the no shutdown command?(part3,step1)

The no shutdown command administratively places the interface in an active state.

5. Which command is used to save the configuration file in RAM to NVRAM?

copy running-config startup-config

# 2.9.1 Basic switch and End Device configuration

## LAYOUT

## IP CONFIGURATION (PC0 & PC1)

## SWITCH CONFIGURATION (S1 & S2)

## CONFIGURATION TESTING (PC0, PC1 & S1, S2)